July 29/87						Stuart LynneSystems File NB. I have split the lines, in the real file they should be one line foreach entry.This entry uses the built in Hayes dialer.van-bc Any a HAYES-40 ATD939-4782 g ogin:--ogin: uuslmac sword:-\c-sword: uuslmac	Connect to van-bc using HAYES dialer with phone number 939-4782 with	protocol g. '-nn' means, that a timeout of nn seconds (here 40) is used	when logging in. Default is 30 seconds.	Wait for ogin: if timeout send \n and wait for ogin:.	Send uuslmac (login ID).	Wait for sword:, if timeout send nothing, wait for sword:	Send uuslamc (login ID).	I use this entry to connect via a DC Hayes Smartmodem 2400, dialing explicity.van-bc Any a DIR 2400 g "" ATZ OK-\d+++\dATZ-OK ATS7=12 OK ATTD939-4782 	CONNECT \d\c ogin:--ogin: uuslmac sword:-\c-sword: uuslmac	Connect to van-bc at 2400 on port a with protocol g. 	Expect nothing, send ATZ to reset the modem, 	Expect OK, if not received send pause +++ pause to try and get	modems attention and wait for OK.	When OK received, send ATS7=12 to shorten connect timeout on modem.	Expect OK, send ATTD939-74782.	Expect CONNECT, send nothing but pause (\c is needed).	Expect ogin:, if not received send newline.	Send login name.	Expect sword:, send password.This entry is used to connect directly to my Callan at 9600. It iscomplicated due to the Callan running a special mgetty program whichthinks it is talking to a Hayes Smartmodem. So we fake it out, thentell it the connection has been made at 9600, then switch to 9600 ourselves.van-bc Any a DIR 2400 g "" OK\r\d\dRING\r\dCONNECT\s9600\d\z9600\ 	ogin:-\r\r-ogin: uuslmac sword:-\c-sword: uuslmac "" \d\d\d\d\d\d\c	Connect to van-bc at 2400 on port a.	Expect nothing, send OK, pause, RING, CONNECT, 9600, 	and change to 9600 bps.	Expect ogin: send login id.	Expect sword: send password.