FAQ: Audio File Formats (version 2.10)

======================================

Table of contents

-----------------

Introduction

Device characteristics

Popular sampling rates

Compression schemes

Current hardware

File formats

File conversions

Playing audio files on UNIX

Playing audio files on micros

The Sound Site Newsletter

Posting sounds

Appendices:

FTP access for non-internet sites

AIFF Format (Audio IFF)

The NeXT/Sun audio file format

IFF/8SVX Format

Playing sound on a PC

The EA-IFF-85 documentation

US Federal Standard 1016 availability

Creative Voice (VOC) file format

RIFF WAVE (.WAV) file format

U-LAW and A-LAW definitions

AVR File Format

Introduction

------------

This is version 2 of this FAQ, which I started in November 1991 under

the name "The audio formats guide".  I bumped the major version number

since the Subject and Newsgroups headers have changed to make the

subject more informative and give the guide a wider audience.  I also

added a Table of contents section at the top.

I am posting this about once a fortnight, either unchanged (just to

inform new readers), or updated (if I learn more or when new hardware

or software becomes popular).  I post to alt.binaries.sounds.{misc,d}

and to comp.dsp, for maximal coverage of people interested in audio,

and to news.answers, for easy reference.

A companion posting with subject "Change to: ..." is occasionally

posted listing the diffs between a new version and the last.  This is

not reposted, and it is suppressed when the diffs are bigger than the

new version.

NEWSFLASH: This FAQ is now also available in distributed hypertext

form!  If you have a WWW browser and direct Internet access you can

point it to "http://voorn.cwi.nl/audio-formats/a00.html".  (WWW is the

CERN World-Wide Web initiative; for more info, telnet or ftp to

info.cern.ch.)

Send updates, comments and questions to <guido@cwi.nl>; flames to

/dev/null.

I'd like to thank everyone who sent me mail with updates for previous

versions.  The list of names is really too long to list you all...

--Guido van Rossum, CWI, Amsterdam <guido@cwi.nl>

"Lobster thermidor aux crevettes with a mornay sauce garnished with

truffle pate, brandy and a fried egg on top and spam"

Device characteristics

----------------------

In this text, I will only use the term "sample" to refer to a single

output value from an A/D converter, i.e., a small integer number

(usually 8 or 16 bits).

Audio data is characterized by the following parameters, which

correspond to settings of the A/D converter when the data was

recorded.  Naturally, the same settings must be used to play the data.

- sampling rate (in samples per second), e.g. 8000 or 44100

- number of bits per sample, e.g. 8 or 16

- number of channels (1 for mono, 2 for stereo, etc.)

Approximate sampling rates are often quoted in Hz or kHz ([kilo-]

Hertz), however, the politically correct term is samples per second

(samples/sec).  Sampling rates are always measured per channel, so for

stereo data recorded at 8000 samples/sec, there are actually 16000

samples in a second.  I will sometimes write 8 k as a shorthand for

8000 samples/sec.

Multi-channel samples are generally interleaved on a frame-by-frame

basis: if there are N channels, the data is a sequence of frames,

where each frame contains N samples, one from each channel.  (Thus,

the sampling rate is really the number of *frames* per second.)  For

stereo, the left channel usually comes first.

The specification of the number of bits for U-LAW (pronounced mu-law

-- the u really stands for the Greek letter mu) samples is somewhat

problematic.  These samples are logarithmically encoded in 8 bits,

like a tiny floating point number; however, their dynamic range is

that of 14 bit linear data.  Source for converting to/from U-LAW

(written by Jef Poskanzer) is distributed as part of the SOX package

mentioned below; it can easily be ripped apart to serve in other

applications.  The official definition is the CCITT standard G.711.

There exists another encoding similar to U-LAW, called A-LAW, which

is used as a European telephony standard.  There is less support for

it in UNIX workstations.

(See the Appendix for some formulae describing U-LAW and A-LAW.)

Popular sampling rates

----------------------

Some sampling rates are more popular than others, for various reasons.

Some recording hardware is restricted to (approximations of) some of

these rates, some playback hardware has direct support for some.  The

popularity of divisors of common rates can be explained by the

simplicity of clock frequency dividing circuits :-).

Samples/sec     Description

5500            One fourth of the Mac sampling rate (rarely seen).

7333            One third of the Mac sampling rate (rarely seen).

8000            Exactly 8000 samples/sec is a telephony standard that

goes together with U-LAW (and also A-LAW) encoding.

Some systems use an slightly different rate; in

particular, the NeXT workstation uses 8012.8210513,

apparently the rate used by Telco CODECs.

11 k            Either 11025, a quarter of the CD sampling rate,

or half the Mac sampling rate (perhaps the most

popular rate on the Mac).

16000           Used by, e.g. the G.722 compression standard.

18.9 k          CD-ROM/XA standard.

22 k            Either 22050, half the CD sampling rate, or the Mac

rate; the latter is precisely 22254.545454545454 but

usually misquoted as 22000.  (Historical note:

22254.5454... was the horizontal scan rate of the

original 128k Mac.)

32000           Used in digital radio, NICAM (Nearly-Instantaneous

Companded Audio Multiplex [IBA/BREMA/BBC]) and other

TV work, at least in the UK; also long play DAT and

Japanese HDTV.

37.8 k          CD-ROM/XA standard for higher quality.

44056           This weird rate is used by professional audio

equipment to fit an integral number of samples in a

video frame.

44100           The CD sampling rate.  (DAT players recording

digitally from CD also use this rate.)

48000           The DAT (Digital Audio Tape) sampling rate for

domestic use.

Files samples on SoundBlaster hardware have sampling rates that are

divisors of 1000000.

While professinal musicians disagree, most people don't have a problem

if recorded sound is played at a slightly different rate, say, 1-2%.

On the other hand, if recorded data is being fed into a playback

device in real time (say, over a network), even the smallest

difference in sampling rate can frustrate the buffering scheme used...

There may be an emerging tendency to standardize on only a few

sampling rates and encoding styles, even if the file formats may

differ.  The suggested rates and styles are:

rate (samp/sec) style mono/stereo

8000 8-bit U-LAW mono

22050 8-bit linear unsigned mono and stereo

44100 16-bit linear signed mono and stereo

Compression schemes

-------------------

Strange though it seems, audio data is remarkably hard to compress

effectively.  For 8-bit data, a Huffman encoding of the deltas between

successive samples is relatively successful.  For 16-bit data,

companies like Sony and Philips have spent millions to develop

proprietary schemes.

Public standards for voice compression are slowly gaining popularity,

e.g. CCITT G.721 and G.723 (ADPCM at 32 and 24 kbits/sec).  (ADPCM ==

Adaptive Delta Pulse Code Modulation.)  Free source code for a *fast*

32 kbits/sec ADPCM algorithm is available by ftp from ftp.cwi.nl as

/pub/adpcm.shar.  (** NOTE: if you are using v1.0, you should get

v1.1, released 17-Dec-1992, which fixes a serious bug -- the quality

of v1.1 is claimed to be better than uLAW **)

There are also two US federal standards, 1016 (Code excited linear

prediction (CELP), 4800 bits/s) and 1015 (LPC-10E, 2400 bits/s).  See

also the appendix for 1016.

(Note that U-LAW and silence detection can also be considered

compression schemes.)

Here's a note about audio codings by Van Jacobson <van@ee.lbl.gov>:

Several people used the words "LPC" and "CELP" interchangably.  They

are very different.  An LPC (Linear Predictive Coding) coder fits

speech to a simple, analytic model of the vocal tract, then throws

away the speech & ships the parameters of the best-fit model.  An LPC

decoder uses those parameters to generate synthetic speech that is

usually more-or-less similar to the original.  The result is

intelligible but sounds like a machine is talking.  A CELP (Code

Excited Linear Predictor) coder does the same LPC modeling but then

computes the errors between the original speech & the synthetic model

and transmits both model parameters and a very compressed

representation of the errors (the compressed representation is an

index into a 'code book' shared between coders & decoders -- this is

why it's called "Code Excited").  A CELP coder does much more work

than an LPC coder (usually about an order of magnitude more) but the

result is much higher quality speech: The FIPS-1016 CELP we're working

on is essentially the same quality as the 32Kb/s ADPCM coder but uses

only 4.8Kb/s (the same as the LPC coder).

Finally, the comp.compression FAQ has some text on the 6:1 audio

compression scheme used by MPEG (a video compression standard-to-be).

It's interesting to note that video compression reaches much higher

ratios (like 26:1).  This FAQ is ftp'able from rtfm.mit.edu

[18.72.1.58] in directory /pub/usenet/news.answers/compression-faq,

files part1 and part2.

Comp.compression also carries a regular posting "How to uncompress

anything" by David Lemson <lemson@uiuc.edu>, which (tersely) hints on

which program you need to uncompress a file whose name ends in .<foo>

for almost any conceivable <foo>.  Ftp'able from ftp.cso.uiuc.edu

(128.174.5.59) in the directory /doc/pcnet as the file compression.

Current hardware

----------------

I am aware of the following computer systems that can play back and

(sometimes) record audio data, with their characteristics.  Note that

for most systems you can also buy "professional" sampling hardware,

which supports much better quality, e.g. >= 44.1 k 16 bits stereo.

The characteristics listed here are a rough estimate of the

capabilities of the basic hardware only (and even here I am on thin

ice, with systems becoming ever more powerful).

machine             bits            max sampling rate    #output channels

Mac                 8               22k                  1

Apple IIgs          8               32k / >70k           8(st)

PC/Soundblaster v1  8               13k / 22k            1

PC/Soundblaster v2  8               15k / 44.1k          1

PC/PAS-16           16              44.1k                ?(st)

Atari ST            8               22k                  1

Atari STe,TT        8               50k                  2

Atari Falcon 030    16              50k                  8(st)

Amiga               8               ~29k                 4(st)

Sun Sparc           U-LAW           8k                   1

Sun Sparcst. 10     U-LAW,8,16      48k                  1(st)

NeXT                U-LAW,8,16      44.1k                1(st)

SGI Indigo          8,16            48k                  4(st)

Acorn Archimedes    ~U-LAW          ~180k                8(st)

Sony RISC-NEWS      8, 16           37.8k                ?(st)

VAXstation 4000     U-LAW           8k                   1

Tandy 1000/*L*      8               22k                  3

HP9000/705,710,425e U,A-LAW,8       8k                   1

4(st) means "four voices, stereo"; sampling rates xx/yy are

different recording/playback rates; *L* is any type with 'L' in it.

All these machines can play back sound without additional hardware,

although the needed software is not always standard; only the Sun,

NeXT and SGI come with standard sampling hardware (the NeXT only

samples U-LAW at 8000 samples/sec from the built-in microphone port;

you need a separate board for other rates).

The new VAXstation 4000 (VLC and model 60) series lets you PLAY audio

(.au) files, and the package DECsound will let you do the recording.

In fact, DECsound is given away free with Motif 1.1 and supports the

VAXstation, Sun SPARCstation, DECvoice, and XMedia audio devices.  Sun

sound files work without change.

The SGI Personal IRIS 4D/30 and 4D/35 have the same capabilities as

the Indigo.

The new Apple Macs have more powerful audio hardware; the latest

models have built-in microphones.

Software exists for the PC that can play sound on its 1-bit speaker

using pulse width modulation (see appendix); the Soundblaster board

records at rates up to 13 k and plays back up to 22 k (weird

combination, but that's the way it is).

Here's some info about the newest Atari machine, the Falcon030.  This

machine has stereo 16 bit CODECs and a 32 MHz Motorola 56001 that can

handle 8 channels of 16 bit audio, up to 50 khz/channel with

simultaneous playback and record.  The Falcon DMA sound engine is also

compatible with the 8 bit stereo DMA used on the STe and TT. All of

these systems use signed data.

On the NeXT, the Motorola 56001 DSP chip is programmable and you can

(in principle) do what you want.  The SGI uses the same DSP chip but

it can't be programmed by users -- SGI prefers to offer it as a shared

system resource to multiple applications, thus enabling developers to

program audio with their Audio Library and avoid code modifications

for execution on future machines with different audio hardware, i.e. a

different DSP.

The Amiga also has a 6-bit volume, which can be used to produce

something like a 14-bit output for each voice.  The hardware can also

use one of each voice-pair to modulate the other in FM (period) or AM

(volume, 6-bits).

The Acorn Archimedes uses a variation on U-LAW with the bit order

reversed and the sign bit in bit 0.  Being a 'minority' architecture,

Arc owners are quite adept at converting sound/image formats from

other machines, and it is unlikely that you'll ever encounter sound in

one of the Arc's own formats (there are several).

CD-I machines form a special category.  The following formats are used:

- PCM 44.1 kHz standard CD format

- ADPCM - Addaptive Delta PCM

- Level A 37.8 kHz 8-bit

- Level B 37.8 kHz 4-bit

- Level C 18.9 kHz 4-bit

File formats

------------

Historically, almost every type of machine used its own file format

for audio data, but some file formats are more generally applicable,

and in general it is possible to define conversions between almost any

pair of file formats -- sometimes losing information, however.

File formats are a separate issue from device characteristics.  There

are two types of file formats: self-describing formats, where the

device parameters and encoding are made explicit in some form of

header, and "raw" formats, where the device parameters and encoding

are fixed.

Self-describing file formats generally define a family of data

encodings, where a header fields indicates the particular encoding

variant used.  Headerless formats define a single encoding and usually

allows no variation in device parameters (except sometimes sampling

rate, which can be a pain to figure out other than by listening to the

sample).

The header of self-describing formats contains the parameters of the

sampling device and sometimes other information (e.g. a

human-readable description of the sound, or a copyright notice).  Most

headers begin with a simple "magic word".  (Some formats do not simply

define a header format, but may contain chunks of data intermingled

with chunks of encoding info.)  The data encoding defines how the

actual samples are stored in the file, e.g. signed or unsigned, as

bytes or short integers, in little-endian or big-endian byte order,

etc.  Strictly spoken, channel interleaving is also part of the

encoding, although so far I have seen little variation in this area.

Some file formats apply some kind of compression to the data, e.g.

Huffman encoding, or simple silence deletion.

Here's an overview of popular file formats.

Self-describing file formats

----------------------------

extension, name   origin          variable parameters (fixed; comments)

.au or .snd       NeXT, Sun       rate, #channels, encoding, info string

.aif(f), AIFF     Apple, SGI      rate, #channels, sample width, lots of info

.aif(f), AIFC     Apple, SGI      same (extension of AIFF with compression)

.iff, IFF/8SVX    Amiga           rate, #channels, instrument info (8 bits)

.voc              Soundblaster    rate (8 bits/1 ch; can use silence deletion)

.wav, WAVE        Microsoft       rate, #channels, sample width, lots of info

.sf               IRCAM           rate, #channels, encoding, info

none, HCOM        Mac             rate (8 bits/1 ch; uses Huffman compression)

none, MIME        Internet        (see below)

.mod or .nst      Amiga           (see below)

Note that the filename extension ".snd" is ambiguous: it can be either

the self-describing NeXT format or the headerless Mac/PC format, or

even a headerless Amiga format.

I know nothing for sure about the origin of HCOM files, only that

there are a lot of them floating around on our system and probably at

FTP sites over the world.  The filenames usually don't have a ".hcom"

extension, but this is what SOX (see below) uses.  The file format

recognized by SOX includes a MacBinary header, where the file

type field is "FSSD".  The data fork begins with the magic word "HCOM"

and contains Huffman compressed data; after decompression it it is 8

bits unsigned data.

IFF/8SVX allows for amplitude contours for sounds (attack/decay/etc).

Compression is optional (and extensible); volume is variable; author,

notes and copyright properties; etc.

AIFF, AIFC and WAVE are similar in spirit but allow more freedom in

encoding style (other than 8 bit/sample), amongst others.

There are other sound formats in use on Amiga by digitizers and music

programs, such as IFF/SMUS.

Appendices describes the NeXT and VOC formats; pointers to more info

about AIFF, AIFC, 8SVX and WAVE (which are too complex to describe

here) are also in appendices.

DEC systems (e.g. DECstation 5000) use a variant of the NeXT format

that uses little-endian encoding and has a different magic number

(0x0064732E in little-endian encoding).

Standard file formats used in the CD-I world are IFF but on the disc

they're in realtime files.

An interesting "interchange format" for audio data is described in the

proposed Internet Standard "MIME", which describes a family of

transport encodings and structuring devices for electronic mail.  This

is an extensible format, and initially standardizes a type of audio

data dubbed "audio/basic", which is 8-bit U-LAW data sampled at 8000

samples/sec.

Finally, a somewhat different but popular format are "MOD" files,

usually with extension ".mod" or ".nst" (they can also have a prefix

of "mod.").  This originated at the Amiga but players now exist for

many platforms.  MOD files are music files containing 2 parts: (1) a

bank of digitized samples; (2) sequencing information describing how

and when to play the samples.  See the appendix "The Amiga MOD Format"

for a description of this file format.

Headerless file formats

-----------------------

extension       origin          parameters

or name

.snd, .fssd     Mac, PC         variable rate, 1 channel, 8 bits unsigned

.ul             US telephony    8 k, 1 channel, 8 bit "U-LAW" encoding

.snd?           Amiga           variable rate, 1 channel, 8 bits signed

It is usually easy to distinguish 8-bit signed formats from unsigned

by looking at the beginning of the data with 'od -b <file | head';

since most sounds start with a little bit of silence containing small

amounts of background noise, the signed formats will have an abundance

of bytes with values 0376, 0377, 0, 1, 2, while the unsigned formats

will have 0176, 0177, 0200, 0201, 0202 instead.  (Using "od -c" will

also show any headers that are tacked in front of the file.)

The Apple IIgs records raw data in the same format as the Mac, but

uses a 0 byte as a terminator; samples with value 0 are replaced by 1.

File conversions

----------------

SOX

---

The most versatile tool for converting between various audio formats

is SOX ("Sound Exchange").  It can read and write various types of

audio files, and optionally applies some special effects (e.g. echo,

channel averaging, or rate conversion).

SOX recognizes all filename extensions listed above except ".snd",

which would be ambiguous anyway, and ".wav" (but there's a patch, see

below).  Use type ".au" for NeXT ".snd" files.  Mac and PC ".snd"

files are completely described by these parameters:

-t raw -b -u -r 11000

(or -r 22000 or -r 7333 or -r 5500; 11000 seems to be the most common

rate).

The source for SOX, version 5, was posted to alt.sources, and should

be widely archived.  To save you the trouble of hunting it down, it

can be gotten by anonymous ftp from wuarchive.wustl.edu, in the

directory usenet/alt.sources/articles, files 5581.Z through 5585.Z.

(These files are compressed news articles containing shar files, if

you hadn't guessed.)  I am sure many sites have similar archives, I'm

just listing one that I know of and which carries a lot of this kind

of stuff.  (Also see the appendix if you don't have Internet access.)

A compressed tar file containing the same version of SOX is available

by anonymous ftp from ftp.cwi.nl [192.16.184.180], in /pub/sox*.tar.Z.

You may be able to locate a nearer version using archie!

Ports of SOX:

- The source as posted should compile on any UNIX system with 4-byte

integers.

- A PC version is available by ftp from ftp.cwi.nl (see above) as

pub/sox4*.zip; also available from the garbo mail server.

- The latest Amiga SOX (corresponding to version 5) is available via

anonymous ftp to wuarchive.wustl.edu, files

systems/amiga/audio/utils/amisox*.  (See below for a non-SOX

solution.)

SOX usage hints:

- Often, the filename extension of sound files posted on the net is

wrong.  Don't give up, try a few other possibilities using the

"-t <type>" option.  Remember that the most common file type is

unsigned bytes, which can be indicated with "-t ub".  You'll have to

guess the proper sampling rate, but often it's 11k or 22k.

- In particular, with SOX version 4 (or earlier), you have to

specify "-t 8svx" for files with an .iff extension.

- When converting linear samples to U-LAW using the .au type for the

output file, you must specify "-U" for the output file, otherwise

you will end up with a file containing a NeXT/Sun header but linear

samples -- only the NeXT will play such files correctly.  Also, you

must explicitly specify an output sampling rate with "-r 8000".

(This may seem fixed for most cases in version 5, but it is still

occasionally necessary, so I'm keeping this warning in.)

Sun Sparc

---------

On Sun Sparcs, starting at SunOS 4.1, a program "raw2audio" is

provided by Sun (in /usr/demo/SOUND -- see below) which takes a raw

U-LAW file and turns it into a ".au" file by prefixing it with an

appropriate header.

NeXT

----

On NeXTs, you can usually rename .au files to .snd and it'll work like

a charm, but some .au files lack header info that the NeXT needs.

This can be fixed by using sndconvert:

sndconvert -c 1 -f 1 -s 8012.8210513 -o nextfile.snd sunfile.au

SGI Indigo and Personal IRIS

----------------------------

SGI supports "soundfiler" (in /usr/sbin), a program similar in

spirit to SOX but with a GUI.  Soundfiler plays aiff, aifc, NeXT/Sun

and .wav formats.  It can do conversions between any of these formats

and to and from raw formats including mulaw.  It also does sample rate

conversions.

Three shell commands are also provided that give the same functionality:

"sfplay", "sfconvert", and "aifcresample" (all in /usr/sbin).

Amiga

-----

Mike Cramer's SoundZAP can do no effects except rate change and it

only does conversions to IFF, but it is generally much faster than

SOX.  (Ftp'able from the same directory as amisox above.)

Tandy

-----

The Tandy 1000 uses a (proprietary?) compressed format.  There is a

PD Mac to Tandy conversion program called CONVERT.

Playing audio files on UNIX

---------------------------

The commands needed to play an audio file depend on the file format

and the available hardware and software.  Most systems can only

directly play sound in their native format; use a conversion program

(see above) to play other formats.

Sun Sparcstation running SunOS 4.x

----------------------------------

Raw U-LAW files can be played using "cat file >/dev/audio".

A whole package for dealing with ".au" files is provided by Sun on an

experimental basis, in /usr/demo/SOUND.  You may have to compile the

programs first.  (If you can't find this directory, either you are not

running SunOS 4.1 yet, or your system administrator hasn't installed

it -- go ask him for it, not me!)  The program "play" in this

directory recognizes all files in Sun/NeXT format, but a SS 1 or 2 can

play only those using U-LAW encoding at 8 k -- the SS 10 hardware

plays other encodings, too.

If you ca't find "play", you can also cat a ".au" file to /dev/audio,

if it uses U-LAW; the header will sound like a short burst of noise

but the rest of the data will sound OK (really, the only difference in

this case between raw U-LAW and ".au" files is the header; the U-LAW

data is exactly the same).

Finally, OpenWindows 3.0 has a full-fledged audio tool.  You can drop

audio file icons into it, edit them, etc.

Sun Sparcstation running Solaris 2.0

------------------------------------

Under SVR4 (and hence Solaris 2.0), writing to /dev/audio from the

shell is a bad idea, because the device driver will flush its queue as

soon as the file is closed.  Use "audioplay" instead.  The supported

formats and sampling rates are the same as above.

NeXT

----

On NeXT machines, the standard "sndplay" program can play all NeXT

format files (this include Sun ".au" files).  It supports at least

U-LAW at 8 k and 16 bits samples at 22 or 44.1 k.  It attempts

on-the-fly conversions for other formats.

Sound files are also played if you double-click on them in the file

browser.

SGI Indigo and Personal IRIS

----------------------------

On SGI Indigo and the 4D/30 and /35 Personal IRIS workstations,

"WorkSpace" plays audio files in .aiff, .aifc, .au, and .wav formats if

you double click them and the sampling rate is one of 8000, 11025,

16000, 22050, 32000, 44100, or 48000.  On the Personal IRIS, you need

to have the audio board installed (check the output from hinv) and you

must run IRIX 3.3.2 or 4.0 or higher.  These files can also be played

with "soundfiler" and "sfplay".  ".aiff" and ".aifc" files at the above

sampling rates can also be played with playaifc.  (All in /usr/sbin)

There is no simple /dev/audio interface on these SGI machines.  (There

was one on 4D/25 machines, reading and writing signed linear 8-bit

samples at rates of 8, 16 and 32 k.)

A program "playulaw" was posted as part of the "radio 2.0" release

that I posted to several source groups recently; it plays raw U-LAW

files on the Indigo or Personal IRIS audio hardware.

Sony NEWS

---------

The Sony RISC-NEWS line (NWS-3250 laptop, NWS-37xx desktop, NWS-38xx

desktop w/ IOP) also has builtin sound capabilities.  You can also buy

external boards for the older NEWS machines or to add extra channels

to the new machines.  In the default mode (8k/8-bit), Sun .au files

are directly supported (you can 'cat' .au files to /dev/sb and have

them play).

Vaxstation 4000

---------------

".au" files can be played by COPYING them to device "SOA0:".  This

device is set up by enabling the driver SODRIVER, as described below:

DEC's sound stuff is like most other new toy.  Hardware first, THEN the

software.   DEC will soon be releasing a layered product called DECsound,

which will let you record, play, and (possibly) manipulate sound files.

Third party product(s) have ALREADY hit the market.

Enabling SODRIVER:   (you can use the following command file)

$!---------------- cut here -------------------------------

$! sound_setup.com    enable SOUND driver

$ run sys$system:sysgen

connect soa0 /adapter=0 /csr=%x0e00 /vector=%o304 /driver=sodriver

exit

$ exit

$!----------------- cut here ------------------------------------

The external audio port comes with a telephone-jack-like port.   For

starters, you can plug a telephone RECEIVER right into this port to

hear your first sound files.   After that, you can use the adapter

(that came with the VaxStation), and plug in a small set of stereo

speakers (the kind you'd plug into a WALKMAN, for example), for more

volume.

Others

------

Most other UNIX boxes don't have audio hardware and thus can't play

audio data.

Playing audio files on micros

-----------------------------

Most micros have at least a speaker built in, so theoretically all you

need is the right software.  Unfortunately most systems don't come

bundled with sound-playing software, so there are many public domain

or shareware software packages, each with their own bugs and features.

Most separate sound recording hardware also comes with playing

software, most of which can play sound (in the file format used by

that hardware) even on machines that don't have that hardware

installed.

Chris S. Craig announces the following software for PCs:

ScopeTrax       This is a complete PC sound player/editor package.  Sounds

can be played back at ANY rate between 1kHz to 65kHz through

the PC speaker or the Sound Blaster.  It supports several

file formats including VOC, IFF/8SVX, raw signed and raw

unsigned.  A separate executable is provided to convert

.au and mu-law to raw format.  ScopeTrax requires EGA/VGA

graphics for editing and displaying sounds on a REALTIME

oscilloscope.  The package also includes:

* An expanded memory player which can play sounds

larger than 640K in size.

* Basic (rough) sound compression/uncompression

utilities.

* Complete documentation.

The package is FREEWARE!  It is available on SIMTEL in the

PD1:[MSDOS.SOUND] directory.

One of the appendices below contains a list of more programs to play

sound on the PC.

For sounds on Atari STs - programs are in the atari/sound/players

directory on atari.archive.umich.edu (141.211.164.8).

Malcolm Slaney from Apple writes:

"We do have tools to play sound back on most of our Unix hosts.  We wrote

a program called TcpPlay that lets us read a sound file on a Unix host,

open a TCP/IP connection to the Mac on my desk, and plays the file.  We

think of it as X windows for sound (at least a step in that direction.)

This software is available for anonymous FTP from ftp.apple.com.

Look for  ~ftp/pub/TcpPlay/TcpPlay.sit.hqx.

Finally, there are MANY tools for working with sound on the Macintosh. Three

applications that come to mind immediately are SoundEdit (formerly by

Farralon and now by MacroMind/Paracomp), Alchemy and Eric Keller's Signalyze.

There are lots of other tools available for sound editing (including some

of the QuickTime Movie tools.)"

On a Tandy 1000, sounds can be played and recorded with DeskMate Sound

(SOUND.PDM), or if they not stored in compressed format, they can also

be played be a program called PLAYSND.  No indication of whether

PLAYSND is PD or not. It hasn't been updated since March of 89.

On the Amiga, OmniPlay by David Champion <dgc3@midway.uchicago.edu>

plays (but does not convert) several sound formats.  It allows Amiga

users to play IFF-8SVX, AIFF, WAV, VOC, .au, .snd, and 8 bit raw

(signed, unsigned, u-law) samples.  OmniPlay is available at the

following FTP sites:

* wuarchive.wustl.edu in /systems/amiga/audio/sampleplayers/oplay121.lha

* amiga.physik.unizh.zh in music/oplay121.lha

The Sound Site Newsletter

-------------------------

An electronic publication with lots of info about digitised sound and

sound formats, albeit mostly on micros, is "The Sound Site

Newsletter".  So far, 8 issues have appeared, the last in January

1992.  Issues can be ftp'ed from saffron.inset.com, directory

directory pub/rogue/newsletters, or from ccb.ucsf.edu,

Pub/Sound_list/Sound.Newsletters.

Posting sounds

--------------

The newsgroup alt.binaries.sounds.misc is dedicated to postings

containing sound.  (Discussions related to such postings belong in

alt.binaries.sounds.d.)

There is no set standard for posting sounds; uuencoded files in most

popular formats are welcome, if split in parts under 50 kBytes.  To

accomodate automatic decoding software (such as the ":decode" command

of the nn newsreader), please place a part indicator of the form

(mm/nn) at the end of your subject meaning this is number mm of a

total of nn part.

It is recommended to post sounds in the format that was used for the

original recording; conversions to other formats often lose

information and would do people with identical hardware as the poster

no favor.  For instance, convering 8-bit linear sound to U-LAW loses

the lower few bits of the data, and rate changing conversions almost

always add noise.  Converting from U-LAW to linear requires expansion

to 16 bit samples if no information loss is allowed!

U-LAW data is best posted with a NeXT/Sun header.

If you have to post a file in a headerless format (usually 8-bit

linear, like ".snd"), please add a description giving at least the

sampling rate and whether the bytes are signed (zero at 0) or unsigned

(zero at 0200).  However, it is highly recommended to add a header

that indicates the sampling rate and encoding scheme; if necessary you

can use SOX to add a header of your choice to raw data.

Compression of sound files usually isn't worth it; the standard

"compress" algorithm doesn't save much when applied to sound data

(typically at most 10-20 percent), and compression algorithms

specifically designed for sound (e.g. NeXT's) are usually

proprietary.  (See also the section "Compression schemes" earlier.)

Appendices

==========

Here are some more detailed pieces of info that I received by e-mail.

They are reproduced here virtually without much editing.

------------------------------------------------------------------------

FTP access for non-internet sites

---------------------------------

>From the sci.space FAQ:

Sites not connected to the Internet cannot use FTP directly, but

there are a few automated FTP servers which operate via email.

Send mail containing only the word HELP to ftpmail@decwrl.dec.com

or bitftp@pucc.princeton.edu, and the servers will send you

instructions on how to make requests

Also:

FAQ lists are available by anonymous FTP from pit-manager.mit.edu

(18.72.1.58) and by email from mail-server@pit-manager.mit.edu (send

a message containing "help" for instructions about the mail server).

------------------------------------------------------------------------

AIFF Format (Audio IFF) and AIFC

--------------------------------

This format was developed by Apple for storing high-quality sampled

sound and musical instrument info; it is also used by SGI and several

professional audio packages (sorry, I know no names).  An extension,

called AIFC or AIFF-C, supports compression (see the last item below).

I've made a BinHex'ed MacWrite version of the AIFF spec (no idea if

it's the same text as mentioned below) available by anonymous ftp from

ftp.cwi.nl [192.16.184.180]; the file is /pub/AudioIFF1.2.hqx.  But

you may be better off with the AIFF-C specs, see below.

Mike Brindley (brindley@ece.orst.edu) writes:

"The complete AIFF spec by Steve Milne, Matt Deatherage (Apple) is

available in 'AMIGA ROM Kernal Reference Manual: Devices (3rd Edition)'

1991 by Commodore-Amiga, Inc.; Addison-Wesley Publishing Co.;

ISBN 0-201-56775-X, starting on page 435 (this edition has a charcoal

grey cover).  It is available in most bookstores, and soon in many

good librairies."

Finally, Mark Callow writes (in comp.sys.sgi):

"I have placed a PostScript version of the AIFF-C specification on

sgi.sgi.com for public ftp.  It is in the file sgi/aiff-c.9.26.91.ps.

sgi.sgi.com's internet host number is (I think) 192.48.153.1."

------------------------------------------------------------------------

The NeXT/Sun audio file format

------------------------------

Here's the complete story on the file format, from the NeXT

documentation.  (Note that the "magic" number is ((int)0x2e736e64),

which equals ".snd".)  Also, at the end, I've added a litte document

that someone posted to the net a couple of years ago, that describes

the format in a bit-by-bit fashion rather than from C.

I received this from Doug Keislar, NeXT Computer.  This is also the

Sun format, except that Sun doesn't recognize as many format codes.  I

added the numeric codes to the table of formats and sorted it.

SNDSoundStruct:  How a NeXT Computer Represents Sound

The NeXT sound software defines the SNDSoundStruct structure to

represent sound.  This structure defines the soundfile and Mach-O

sound segment formats and the sound pasteboard type.  It's also used

to describe sounds in Interface Builder.  In addition, each instance

of the Sound Kit's Sound class encapsulates a SNDSoundStruct and

provides methods to access and modify its attributes.

Basic sound operations, such as playing, recording, and cut-and-paste

editing, are most easily performed by a Sound object.  In many cases,

the Sound Kit obviates the need for in-depth understanding of the

SNDSoundStruct architecture.  For example, if you simply want to

incorporate sound effects into an application, or to provide a simple

graphic sound editor (such as the one in the Mail application), you

needn't be aware of the details of the SNDSoundStruct.  However, if

you want to closely examine or manipulate sound data you should be

familiar with this structure.

The SNDSoundStruct contains a header, information that describes the

attributes of a sound, followed by the data (usually samples) that

represents the sound.  The structure is defined (in

sound/soundstruct.h) as:

typedef struct {

int magic;               /* magic number SND_MAGIC */

int dataLocation;        /* offset or pointer to the data */

int dataSize;            /* number of bytes of data */

int dataFormat;          /* the data format code */

int samplingRate;        /* the sampling rate */

int channelCount;        /* the number of channels */

char info[4];            /* optional text information */

} SNDSoundStruct;

SNDSoundStruct Fields

magic

magic is a magic number that's used to identify the structure as a

SNDSoundStruct.  Keep in mind that the structure also defines the

soundfile and Mach-O sound segment formats, so the magic number is

also used to identify these entities as containing a sound.

dataLocation

It was mentioned above that the SNDSoundStruct contains a header

followed by sound data.  In reality, the structure only contains the

header; the data itself is external to, although usually contiguous

with, the structure.  (Nonetheless, it's often useful to speak of the

SNDSoundStruct as the header and the data.)  dataLocation is used to

point to the data.  Usually, this value is an offset (in bytes) from

the beginning of the SNDSoundStruct to the first byte of sound data.

The data, in this case, immediately follows the structure, so

dataLocation can also be thought of as the size of the structure's

header.  The other use of dataLocation, as an address that locates

data that isn't contiguous with the structure, is described in

"Format Codes," below.

dataSize, dataFormat, samplingRate, and channelCount

These fields describe the sound data.

dataSize is its size in bytes (not including the size of the

SNDSoundStruct).

dataFormat is a code that identifies the type of sound.  For sampled

sounds, this is the quantization format.  However, the data can also

be instructions for synthesizing a sound on the DSP.  The codes are

listed and explained in "Format Codes," below.

samplingRate is the sampling rate (if the data is samples).  Three

sampling rates, represented as integer constants, are supported by

the hardware:

Constant        Sampling Rate (samples/sec)

SND_RATE_CODEC  8012.821        (CODEC input)

SND_RATE_LOW    22050.0 (low sampling rate output)

SND_RATE_HIGH   44100.0 (high sampling rate output)

channelCount is the number of channels of sampled sound.

info

info is a NULL-terminated string that you can supply to provide a

textual description of the sound.  The size of the info field is set

when the structure is created and thereafter can't be enlarged.  It's

at least four bytes long (even if it's unused).

Format Codes

A sound's format is represented as a positive 32-bit integer.  NeXT

reserves the integers 0 through 255; you can define your own format

and represent it with an integer greater than 255.  Most of the

formats defined by NeXT describe the amplitude quantization of

sampled sound data:

Value   Code    Format

0       SND_FORMAT_UNSPECIFIED  unspecified format

1       SND_FORMAT_MULAW_8      8-bit mu-law samples

2       SND_FORMAT_LINEAR_8     8-bit linear samples

3       SND_FORMAT_LINEAR_16    16-bit linear samples

4       SND_FORMAT_LINEAR_24    24-bit linear samples

5       SND_FORMAT_LINEAR_32    32-bit linear samples

6       SND_FORMAT_FLOAT        floating-point samples

7       SND_FORMAT_DOUBLE       double-precision float samples

8       SND_FORMAT_INDIRECT     fragmented sampled data

9       SND_FORMAT_NESTED       ?

10      SND_FORMAT_DSP_CORE     DSP program

11      SND_FORMAT_DSP_DATA_8   8-bit fixed-point samples

12      SND_FORMAT_DSP_DATA_16  16-bit fixed-point samples

13      SND_FORMAT_DSP_DATA_24  24-bit fixed-point samples

14      SND_FORMAT_DSP_DATA_32  32-bit fixed-point samples

15      ?

16      SND_FORMAT_DISPLAY      non-audio display data

17      SND_FORMAT_MULAW_SQUELCH        ?

18      SND_FORMAT_EMPHASIZED   16-bit linear with emphasis

19      SND_FORMAT_COMPRESSED   16-bit linear with compression

20      SND_FORMAT_COMPRESSED_EMPHASIZED        A combination of the two above

21      SND_FORMAT_DSP_COMMANDS Music Kit DSP commands

22      SND_FORMAT_DSP_COMMANDS_SAMPLES         ?

Most formats identify different sizes and types of

sampled data.  Some deserve special note:

--      SND_FORMAT_DSP_CORE format contains data that represents a

loadable DSP core program.  Sounds in this format are required by the

SNDBootDSP() and SNDRunDSP() functions.  You create a

SND_FORMAT_DSP_CORE sound by reading a DSP load file (extension

".lod") with the SNDReadDSPfile() function.

--      SND_FORMAT_DSP_COMMANDS is used to distinguish sounds that

contain DSP commands created by the Music Kit.  Sounds in this format

can only be created through the Music Kit's Orchestra class, but can

be played back through the SNDStartPlaying() function.

--      SND_FORMAT_DISPLAY format is used by the Sound Kit's

SoundView class.  Such sounds can't be played.

--      SND_FORMAT_INDIRECT indicates data that has become

fragmented, as described in a separate section, below.

--      SND_FORMAT_UNSPECIFIED is used for unrecognized formats.

Fragmented Sound Data

Sound data is usually stored in a contiguous block of memory.

However, when sampled sound data is edited (such that a portion of

the sound is deleted or a portion inserted), the data may become

discontiguous, or fragmented.  Each fragment of data is given its own

SNDSoundStruct header; thus, each fragment becomes a separate

SNDSoundStruct structure.  The addresses of these new structures are

collected into a contiguous, NULL-terminated block; the dataLocation

field of the original SNDSoundStruct is set to the address of this

block, while the original format, sampling rate, and channel count

are copied into the new SNDSoundStructs.

Fragmentation serves one purpose:  It avoids the high cost of moving

data when the sound is edited.  Playback of a fragmented sound is

transparent-you never need to know whether the sound is fragmented

before playing it.  However, playback of a heavily fragmented sound

is less efficient than that of a contiguous sound.  The

SNDCompactSamples() C function can be used to compact fragmented

sound data.

Sampled sound data is naturally unfragmented.  A sound that's freshly

recorded or retrieved from a soundfile, the Mach-O segment, or the

pasteboard won't be fragmented.  Keep in mind that only sampled data

can become fragmented.

_________________________

>From mentor.cc.purdue.edu!purdue!decwrl!ucbvax!ziploc!eps Wed Apr  4

23:56:23 EST 1990

Article 5779 of comp.sys.next:

Path: mentor.cc.purdue.edu!purdue!decwrl!ucbvax!ziploc!eps

>From: eps@toaster.SFSU.EDU (Eric P. Scott)

Newsgroups: comp.sys.next

Subject: Re: Format of NeXT sndfile headers?

Message-ID: <445@toaster.SFSU.EDU>

Date: 31 Mar 90 21:36:17 GMT

References: <14978@phoenix.Princeton.EDU>

Reply-To: eps@cs.SFSU.EDU (Eric P. Scott)

Organization: San Francisco State University

Lines: 42

In article <14978@phoenix.Princeton.EDU>

bskendig@phoenix.Princeton.EDU (Brian Kendig) writes:

>I'd like to take a program I have that converts Macintosh sound

files

>to NeXT sndfiles and polish it up a bit to go the other direction as

>well.

Two people have already submitted programs that do this

(Christopher Lane and Robert Hood); check the various

NeXT archive sites.

>       Could someone please give me the format of a NeXT sndfile

>header?

"big-endian"

0       1       2       3

+-------+-------+-------+-------+

0       | 0x2e  | 0x73  | 0x6e  | 0x64  |       "magic" number

+-------+-------+-------+-------+

4       |                               |       data location

+-------+-------+-------+-------+

8       |                               |       data size

+-------+-------+-------+-------+

12      |                               |       data format (enum)

+-------+-------+-------+-------+

16      |                               |       sampling rate (int)

+-------+-------+-------+-------+

20      |                               |       channel count

+-------+-------+-------+-------+

24      |       |       |       |       |       (optional) info

string

28 = minimum value for data location

data format values can be found in /usr/include/sound/soundstruct.h

Most common combinations:

sampling  channel    data

rate    count  format

voice file   8012        1       1 =  8-bit mu-law

system beep 22050        2       3 = 16-bit linear

CD-quality  44100        2       3 = 16-bit linear

------------------------------------------------------------------------

IFF/8SVX Format

---------------

Newsgroups: alt.binaries.sounds.d,alt.sex.sounds

Subject: Format of the IFF header (Amiga sounds)

Message-ID: <2509@tardis.Tymnet.COM>

From: jms@tardis.Tymnet.COM (Joe Smith)

Date: 23 Oct 91 23:54:38 GMT

Followup-To: alt.binaries.sounds.d

Organization: BT North America (Tymnet)

The first 12 bytes of an IFF file are used to distinguish between an Amiga

picture (FORM-ILBM), an Amiga sound sample (FORM-8SVX), or other file

conforming to the IFF specification.  The middle 4 bytes is the count of

bytes that follow the "FORM" and byte count longwords.  (Numbers are stored

in M68000 form, high order byte first.)

------------------------------------------

FutureSound audio file, 15000 samples at 10.000KHz, file is 15048 bytes long.

0000: 464F524D 00003AC0 38535658 56484452    FORM..:.8SVXVHDR

F O R M     15040 8 S V X  V H D R

0010: 00000014 00003A98 00000000 00000000    ......:.........

20    15000        0        0

0020: 27100100 00010000 424F4459 00003A98    '.......BODY..:.

10000 1 0    1.0   B O D Y     15000

0000000..03 = "FORM", identifies this as an IFF format file.

FORM+00..03 (ULONG) = number of bytes that follow.  (Unsigned long int.)

FORM+03..07 = "8SVX", identifies this as an 8-bit sampled voice.

????+00..03 = "VHDR", Voice8Header, describes the parameters for the BODY.

VHDR+00..03 (ULONG) = number of bytes to follow.

VHDR+04..07 (ULONG) = samples in the high octave 1-shot part.

VHDR+08..0B (ULONG) = samples in the high octave repeat part.

VHDR+0C..0F (ULONG) = samples per cycle in high octave (if repeating), else 0.

VHDR+10..11 (UWORD) = samples per second.  (Unsigned 16-bit quantity.)

VHDR+12     (UBYTE) = number of octaves of waveforms in sample.

VHDR+13     (UBYTE) = data compression (0=none, 1=Fibonacci-delta encoding).

VHDR+14..17 (FIXED) = volume.  (The number 65536 means 1.0 or full volume.)

????+00..03 = "BODY", identifies the start of the audio data.

BODY+00..03 (ULONG) = number of bytes to follow.

BODY+04..NNNNN      = Data, signed bytes, from -128 to +127.

0030: 04030201 02030303 04050605 05060605

0040: 06080806 07060505 04020202 01FF0000

0050: 00000000 FF00FFFF FFFEFDFD FDFEFFFF

0060: FDFDFF00 00FFFFFF 00000000 00FFFF00

0070: 00000000 00FF0000 00FFFEFF 00000000

0080: 00010000 000101FF FF0000FE FEFFFFFE

0090: FDFDFEFD FDFFFFFC FDFEFDFD FEFFFEFE

00A0: FFFEFEFE FEFEFEFF FFFFFEFF 00FFFF01

This small section of the audio sample shows the number ranging from -5 (0xFD)

to +8 (0x08).  Warning: Do not assume that the BODY starts 48 bytes into the

file.  In addition to "VHDR", chunks labeled "NAME", "AUTH", "ANNO", or

"(c) " may be present, and may be in any order.  You will have to check the

byte count in each chunk to determine how many bytes to skip.

------------------------------------------------------------------------

Playing sound on a PC

---------------------

From: Eric A Rasmussen

Any turbo PC (8088 at 8 Mhz or greater)/286/386/486/etc. can produce a quality

playback of single channel 8 bit sounds on the internal (1 bit, 1 channel)

speaker by utilizing Pulse-Width-Modulation, which toggles the speaker faster

than it can physically move to simulate positions between fully on and fully

off.  There are several PD programs of this nature that I know of:

REMAC  - Plays MAC format sound files.  Files on the Macintosh, at least the

sound files that I've ripped apart, seem to contain 3 parts.  The

first two are info like what the file icon looks like and other

header type info.  The third part contains the raw sample data, and

it is this portion of the file which is saved to a seperate file,

often named with the .snd extension by PC users.  Personally, I like

to name the files .s1, .s2, .s3, or .s4 to indicate the sampling rate

of the file. (-s# is how to specify the playback rate in REMAC.)

REMAC provides playback rates of 5550hz, 7333hz, 11 khz, & 22 khz.

REMAC2 - Same as REMAC, but sounds better on higher speed machines.

REPLAY - Basically same as REMAC, but for playback of Atari ST sounds.

Apparently, the Atari has two sound formats, one of which sounds like

garbage if played by REMAC or REPLAY in the incorrect mode.  The

other file format works fine with REMAC and so appears to be 'normal'

unsigned 8-bit data.  REPLAY provides playback rates of 11.5 khz,

12.5 khz, 14 khz, 16 khz, 18.5 khz, 22khz, & 27 khz.

These three programs are all by the same author, Richard E. Zobell who does

not have an internet mail address to my knowledge, but does have a GEnie email

address of R.ZOBELL.

Additionally, there are various stand-alone demos which use the internal

speaker, of which there is one called mushroom which plays a 30 second

advertising jingle for magic mushroom room deoderizers which is pretty

humerous.  I've used this player to playback samples that I ripped out of the

commercial game program Mean Streets, which uses something they call RealSound

(tm) to playback digital samples on the internal speaker. (Of course, I only do

this on my own system, and since I own the game, I see no problems with it.)

For owners of 8 Mhz 286's and above, the option to play 4 channel 8 bit sounds

(with decent quality) on the internal speaker is also a reality.  Quite a

number of PD programs exist to do this, including, but not limited to:

ModEdit, ModPlay, ScreamTracker, STM, Star Trekker, Tetra, and probably a few

more.

All these programs basically make use of various sound formats used by the

Amiga line of computers.  These include .stm files, .mod files

[a.k.a. mod. files], and .nst files [really the same hing].  Also,

these programs pretty much all have the option to playback the

sound to add-on hardware such as the SoundBlaster card, the Covox series of

devices, and also to direct the data to either one or two (for stereo)

parallel ports, which you could attach your own D/A's to.  (From what I have

seen, the Covox is basically an small amplified speaker with a D/A which plugs

into the parallel port.  This sounds very similiar to the Disney Sound System

(DSS) which people have been talking about recently.)

------------------------------------------------------------------------

The EA-IFF-85 documentation

---------------------------

From: dgc3@midway.uchicago.edu

As promised, here's an ftp location for the EA-IFF-85 documentation.  It's

the November 1988 release as revised by Commodore (the last public release),

with specifications for IFF FORMs for graphics, sound, formatted text, and

more.  IFF FORMS now exist for other media, including structured drawing, and

new documentation is now available only from Commodore.

The documentation is at grind.isca.uiowa.edu [128.255.19.233], in the

directory /amiga/f1/ff185.  The complete file list is as follows:

DOCUMENTS.zoo

EXAMPLES.zoo

EXECUTABLE.zoo

INCLUDE.zoo

LINKER_INFO.zoo

OBJECT.zoo

SOURCE.zoo

TP_IFF_Specs.zoo

All files except DOCUMENTS.zoo are Amiga-specific, but may be used as a basis

for conversion to other platforms.  Well, I take that tentatively back.  I

don't know what TP_IFF_Specs.zoo contains, so it might be non-Amiga-specific.

------------------------------------------------------------------------

US Federal Standard 1016 availability

-------------------------------------

From: Joe Campbell  N3JBC  jpcampb@afterlife.ncsc.mil  74040.305@compuserve.com

The U.S. DoD's Federal-Standard-1016 4800 bps code excited linear prediction

voice coder version 3.2 (CELP 3.2) Fortran and C simulation source codes are

now available for worldwide distribution at no charge (on DOS diskettes,

but configured to compile on Sun SPARC stations) from:

Bob Fenichel

National Communications System

Washington, D.C.  20305

1-703-692-2124

1-703-746-4960 (fax)

In addition to the source codes, example input and processed speech files

are included along with a technical information bulletin to assist in

implementation of FS-1016 CELP.  (An anonymous ftp site is being considered

for future releases.)

Copies of the FS-1016 document are available for $2.50 each from:

GSA Rm 6654

7th & D St SW

Washington, D.C.  20407

1-202-708-9205

The following articles describe the Federal-Standard-1016 4.8-kbps CELP

coder (it's unnecessary to read more than one):

Campbell, Joseph P. Jr., Thomas E. Tremain and Vanoy C. Welch,

"The Federal Standard 1016 4800 bps CELP Voice Coder," Digital Signal

Processing, Academic Press, 1991, Vol. 1, No. 3, p. 145-155.

Campbell, Joseph P. Jr., Thomas E. Tremain and Vanoy C. Welch,

"The DoD 4.8 kbps Standard (Proposed Federal Standard 1016),"

in Advances in Speech Coding, ed. Atal, Cuperman and Gersho,

Kluwer Academic Publishers, 1991, Chapter 12, p. 121-133.

Campbell, Joseph P. Jr., Thomas E. Tremain and Vanoy C. Welch, "The

Proposed Federal Standard 1016 4800 bps Voice Coder:  CELP," Speech

Technology Magazine, April/May 1990, p. 58-64.

For U.S. FED-STD-1016 (4800 bps CELP) _realtime_ DSP code

and information about products using this code, contact:

John DellaMorte

DSP Software Engineering

165 Middlesex Tpk, Suite 206

Bedford, MA  01730

1-617-275-3733

1-617-275-4323 (fax)

dspse.bedford@channel1.com

DSP Software Engineering's code can run on a DSP Research's Tiger 30 board

(a PC board with a TMS320C3x and analog interface suited to development work)

or on Intellibit's AE2000 TMS320C31 based 3" by 2.5" card.

DSP Research                Intellibit

1095 E. Duane Ave.          P.O. Box 9785

Sunnyvale, CA  94086        McLean, VA  22102-0785

(408)773-1042               (703)442-4781

(408)736-3451 (fax)         (703)442-4784 (fax)

From: tobiasr@monolith.lrmsc.loral.com (Richard Tobias )

For U.S. FED-STD-1016 (4800 bps CELP) _realtime_ DSP code and

information about products using this code using the AT&T DSP32C and

AT&T DSP3210, contact:

White Eagle Systems Technology, Inc.

1123 Queensbridge Way

San Jose, CA 95120

(408) 997-2706

(408) 997-3584 (fax)

rjjt@netcom.com

------------------------------------------------------------------------

Creative Voice (VOC) file format

--------------------------------

From: galt@dsd.es.com

(byte numbers are hex!)

HEADER (bytes 00-19)

Series of DATA BLOCKS (bytes 1A+) [Must end w/ Terminator Block]

- ---------------------------------------------------------------

HEADER:

=======

byte #     Description

------     ------------------------------------------

00-12      "Creative Voice File"

13         1A (eof to abort printing of file)

14-15      Offset of first datablock in .voc file (std 1A 00

in Intel Notation)

16-17      Version number (minor,major) (VOC-HDR puts 0A 01)

18-19      2's Comp of Ver. # + 1234h (VOC-HDR puts 29 11)

- ---------------------------------------------------------------

DATA BLOCK:

===========

Data Block:  TYPE(1-byte), SIZE(3-bytes), INFO(0+ bytes)

NOTE: Terminator Block is an exception -- it has only the TYPE byte.

TYPE   Description     Size (3-byte int)   Info

----   -----------     -----------------   -----------------------

00     Terminator      (NONE)              (NONE)

01     Sound data      2+length of data    *

02     Sound continue  length of data      Voice Data

03     Silence         3                   **

04     Marker          2                   Marker# (2 bytes)

05     ASCII           length of string    null terminated string

06     Repeat          2                   Count# (2 bytes)

07     End repeat      0                   (NONE)

*Sound Info Format:       **Silence Info Format:

---------------------      ----------------------------

00   Sample Rate           00-01  Length of silence - 1

01   Compression Type      02     Sample Rate

02+  Voice Data

Marker#           -- Driver keeps the most recent marker in a status byte

Count#            -- Number of repetitions + 1

Count# may be 1 to FFFE for 0 - FFFD repetitions

or FFFF for endless repetitions

Sample Rate       -- SR byte = 256-(1000000/sample_rate)

Length of silence -- in units of sampling cycle

Compression Type  -- of voice data

8-bits    = 0

4-bits    = 1

2.6-bits  = 2

2-bits    = 3

Multi DAC = 3+(# of channels) [interesting--

this isn't in the developer's manual]

------------------------------------------------------------------------

RIFF WAVE (.WAV) file format

----------------------------

RIFF is a format by Microsoft and IBM which is similar in spirit and

functionality as EA-IFF-85, but not compatible (and it's in

little-endian byte order, of course :-).  WAVE is RIFF's equivalent of

AIFF, and its inclusion in Microsoft Windows 3.1 has suddenly made it

important to know about.

Rob Ryan was kind enough to send me a description of the RIFF format.

Unfortunately, it is too big to include here (27 k), but I've made it

available for anonymous ftp as ftp.cwi.nl:/pub/RIFF-format.

And here's a pointer to the official description from Matt Saettler,

Microsoft Multimedia:

"The complete definition of the WAVE file format as defined by

IBM/Microsoft is available for anon. FTP from ftp.uu.net in the

vendor/microsoft/multimedia directory."

(Rob Ryan's version may actually be an extract from one of the files

stored there.)

------------------------------------------------------------------------

U-LAW and A-LAW definitions

---------------------------

[Adapted from information provided by duggan@cc.gatech.edu (Rick

Duggan) and davep@zenobia.phys.unsw.EDU.AU (David Perry)]

u-LAW (really mu-LAW) is

sgn(m)   (     |m |)       |m |

y=    ------- ln( 1+ u|--|)       |--| =< 1

ln(1+u)   (     |mp|)       |mp|

A-LAW is

|     A    (m )                 |m |    1

|  ------- (--)                 |--| =< -

|  1+ln A  (mp)                 |mp|    A

y=|

| sgn(m) (        |m |)    1    |m |

| ------ ( 1+ ln A|--|)    - =< |--| =< 1

| 1+ln A (        |mp|)    A    |mp|

Values of u=100 and 255, A=87.6, mp is the Peak message value, m is

the current quantised message value.  (The formulae get simpler if you

substitute x for m/mp and sgn(x) for sgn(m); then -1 <= x <= 1.)

Converting from u-LAW to A-LAW is in a sense "lossy" since there are

quantizing errors introduced in the conversion.

"..the u-LAW used in North America and Japan, and the

A-LAW used in Europe and the rest of the world and

international routes.."

References:

Modern Digital and Analog Communication Systems, B.P.Lathi., 2nd ed.

ISBN 0-03-027933-X

Transmission Systems for Communications

Fifth Edition

by Members of the Technical Staff at Bell Telephone Laboratories

Bell Telephone Laboratories, Incorporated

Copyright 1959, 1964, 1970, 1982

------------------------------------------------------------------------

AVR File Format

---------------

From: hyc@hanauma.Jpl.Nasa.Gov (Howard Chu)

A lot of PD software exists to play Mac .snd files on the ST. One other

format that seems pretty popular (used by a number of commercial packages)

is the AVR format (from Audio Visual Research). This format has a 128 byte

header that looks like this:

char magic[4]="2BIT";

char name[8];           /* null-padded sample name */

short mono;             /* 0 = mono, 0xffff = stereo */

short rez;              /* 8 = 8 bit, 16 = 16 bit */

short sign;             /* 0 = unsigned, 0xffff = signed */

short loop;             /* 0 = no loop, 0xffff = looping sample */

short midi;             /* 0xffff = no MIDI note assigned,

0xffXX = single key note assignment

0xLLHH = key split, low/hi note */

long rate;              /* sample frequency in hertz */

long size;              /* sample length in bytes or words (see rez) */

long lbeg;              /* offset to start of loop in bytes or words.

set to zero if unused. */

long lend;              /* offset to end of loop in bytes or words.

set to sample length if unused. */

short res1;             /* Reserved, MIDI keyboard split */

short res2;             /* Reserved, sample compression */

short res3;             /* Reserved */

char ext[20];           /* Additional filename space, used

if (name[7] != 0) */

char user[64];          /* User defined. Typically ASCII message. */

-----------------------------------------------------------------------

The Amiga MOD Format

--------------------

From: norlin@mailhost.ecn.uoknor.edu (Norman Lin)

MOD files are music files containing 2 parts:

(1) a bank of digitized samples

(2) sequencing information describing how and when to play the samples

MOD files originated on the Amiga, but because of their flexibility

and the extremely large number of MOD files available, MOD players

are now available for a variety of machines (IBM PC, Mac, Sparc

Station, etc.)

The samples in a MOD file are raw, 8 bit, signed, headerless, linear

digital data.  There may be up to 31 distinct samples in a MOD file,

each with a length of up to 128K (though most are much smaller; say,

10K - 60K).  An older MOD format only allowed for up to 15 samples in

a MOD file; you don't see many of these anymore.  There is no standard

sampling rate for these samples.

The sequencing information in a MOD file contains 4 tracks of

information describing which, when, for how long, and at what frequency

samples should be played.  This means that a MOD file can have up

to 31 distinct (digitized) instrument sounds, with up to 4 playing

simultaneously at any given point.  This allows a wide variety

of orchestrational possibilities, including use of voice samples

or creation of one's own instruments (with appropriate sampling

hardware/software).  The ability to use one's own samples as instruments

is a flexibility that other music files/formats do not share, and

is one of the reasons MOD files are so popular, numerous, and diverse.

15 instrument MODs, as noted above, are somewhat older than 31

instrument MODs and are not (at least not by me) seen very often

anymore.  Their format is identical to that of 31 instrument MODs

except:

(1) Since there are only 15 samples, the information for the last (15th)

sample starts at byte 440 and goes through byte 469.

(2) The songlength is at byte 470 (contrast with byte 950 in 31 instrument

MOD)

(3) Byte 471 appears to be ignored, but has been observed to be 127.

(Sorry, this is from observation only)

(4) Byte 472 begins the pattern sequence table (contrast with byte 952

in a 31 instrument MOD)

(5) Patterns start at byte 600 (contrast with byte 1084 in 31 instrument MOD)

"ProTracker," an Amiga MOD file creator/editor, is available for ftp

everywhere as pt??.lzh.

From: Apollo Wong <apollo@ee.ualberta.ca>

From: M.J.H.Cox@bradford.ac.uk (Mark Cox)

Newsgroups: alt.sb.programmer

Subject: Re: Format for MOD files...

Message-ID: <1992Mar18.103608.4061@bradford.ac.uk>

Date: 18 Mar 92 10:36:08 GMT

Organization: University of Bradford, UK

wdc50@DUTS.ccc.amdahl.com (Winthrop D Chan) writes:

>I'd like to know if anyone has a reference document on the format of the

>Amiga Sound/NoiseTracker (MOD) files. The author of Modplay said he was going

>to release such a document sometime last year, but he never did. If anyone

I found this one, which covers it better than I can explain it - if you

use this in conjunction with the documentation that comes with Norman

Lin's Modedit program it should pretty much cover it.

Mark J Cox

/***********************************************************************

Protracker 1.1B Song/Module Format:

-----------------------------------

Offset  Bytes  Description

------  -----  -----------

0     20    Songname. Remember to put trailing null bytes at the end...

Information for sample 1-31:

Offset  Bytes  Description

------  -----  -----------

20     22    Samplename for sample 1. Pad with null bytes.

42      2    Samplelength for sample 1. Stored as number of words.

Multiply by two to get real sample length in bytes.

44      1    Lower four bits are the finetune value, stored as a signed

four bit number. The upper four bits are not used, and

should be set to zero.

Value:  Finetune:

0        0

1       +1

2       +2

3       +3

4       +4

5       +5

6       +6

7       +7

8       -8

9       -7

A       -6

B       -5

C       -4

D       -3

E       -2

F       -1

45      1    Volume for sample 1. Range is $00-$40, or 0-64 decimal.

46      2    Repeat point for sample 1. Stored as number of words offset

from start of sample. Multiply by two to get offset in bytes.

48      2    Repeat Length for sample 1. Stored as number of words in

loop. Multiply by two to get replen in bytes.

Information for the next 30 samples starts here. It's just like the info for

sample 1.

Offset  Bytes  Description

------  -----  -----------

50     30    Sample 2...

80     30    Sample 3...

.

.

.

890     30    Sample 30...

920     30    Sample 31...

Offset  Bytes  Description

------  -----  -----------

950      1    Songlength. Range is 1-128.

951      1    Well... this little byte here is set to 127, so that old

trackers will search through all patterns when loading.

Noisetracker uses this byte for restart, but we don't.

952    128    Song positions 0-127. Each hold a number from 0-63 that

tells the tracker what pattern to play at that position.

1080      4    The four letters "M.K." - This is something Mahoney & Kaktus

inserted when they increased the number of samples from

15 to 31. If it's not there, the module/song uses 15 samples

or the text has been removed to make the module harder to

rip. Startrekker puts "FLT4" or "FLT8" there instead.

Offset  Bytes  Description

------  -----  -----------

1084    1024   Data for pattern 00.

.

.

.

xxxx  Number of patterns stored is equal to the highest patternnumber

in the song position table (at offset 952-1079).

Each note is stored as 4 bytes, and all four notes at each position in

the pattern are stored after each other.

00 -  chan1  chan2  chan3  chan4

01 -  chan1  chan2  chan3  chan4

02 -  chan1  chan2  chan3  chan4

etc.

Info for each note:

_____byte 1_____   byte2_    _____byte 3_____   byte4_

/                \ /      \  /                \ /      \

0000          0000-00000000  0000          0000-00000000

Upper four    12 bits for    Lower four    Effect command.

bits of sam-  note period.   bits of sam-

ple number.                  ple number.

Periodtable for Tuning 0, Normal

C-1 to B-1 : 856,808,762,720,678,640,604,570,538,508,480,453

C-2 to B-2 : 428,404,381,360,339,320,302,285,269,254,240,226

C-3 to B-3 : 214,202,190,180,170,160,151,143,135,127,120,113

To determine what note to show, scan through the table until you find

the same period as the one stored in byte 1-2. Use the index to look

up in a notenames table.

This is the data stored in a normal song. A packed song starts with the

four letters "PACK", but i don't know how the song is packed: You can

get the source code for the cruncher/decruncher from us if you need it,

but I don't understand it; I've just ripped it from another tracker...

In a module, all the samples are stored right after the patterndata.

To determine where a sample starts and stops, you use the sampleinfo

structures in the beginning of the file (from offset 20). Take a look

at the mt_init routine in the playroutine, and you'll see just how it

is done.

Lars "ZAP" Hamre/Amiga Freelancers

***********************************************************************/

--

Mark J Cox -----

Bradford, UK ---

-----------------------------------------------------------------------